Outdoor Wireless 11ac Device

Model: GNT-CP1500

Short Specification

GNT-CP1500 is a high-performance, high-speed outdoor long-distance wireless bridge with characteristics such as long transmission distance, strong penetration ability, and strong anti-interference ability.

Product Highlight:

- Support non-standard frequency points, spread spectrum to 4.9-6.1GHz

- Integrated 23dBi dual polarization dish antenna, up to 15 kilometers

- TDMA Intelligent Dynamic Polling Protocol

- Support 5, 10MHz narrowband operating bandwidth

- Supporting IPv6, ATPC, VLAN

Hardware Specification

| Model | GNT-CP1500 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CPU | QCA9563+QCA9882+QCA8334 |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| DDR2 | 64MB |  |  |  |  |
| 5G Band | 5.150GHz～5.850GHz |  |  |  |  |
| 5G WIFI Protocol | 802.11 a / n /ac |  |  |  |  |
| Maximum rate | 900Mbps |  |  |  |  |
| Antenna | Integrated 23dBi dual polarization dish antenna |  |  |  |  |
| 5G Tx power | 802.11a | 54M | 23±2dBm | 6M | 26±2dBm |
|  | 802.11n HT20 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11n HT40 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT20 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
| 5G Receiving sensitivity | 802.11ac VHT40 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT80 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11a | 54M | -75dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -72dBm | MCS0 | -90dBm |
| 5G EVM | 802.11a_54M: ≤-25 dB；802.11n_MCS7: ≤-28 dB；802.11AC_MCS9:≤-32dB |  |  |  |  |
| frequency deviation（ppm） | ±20ppm |  |  |  |  |
| Port (WAN) | 1*10/100/1000M WAN, support 802.3af 48V POE |  |  |  |  |
| Port（LAN） | 1*10/100/1000M LAN |  |  |  |  |
| Supply | 802.3af，DC 12~24V |  |  |  |  |
| Consumption | <10W |  |  |  |  |
| Reset Button | Long press for 6-10 seconds to restore factory default configuration |  |  |  |  |
| LED | SYS, WAN, LAN, signal*5 |  |  |  |  |
| Size | 374.2mm*201mm |  |  |  |  |
| Weight | 2.6KG |  |  |  |  |
| Environment | Operating temperature: -40°C to 70°C Storage temperature: -40°C to 70°C；humidity：5％～95％（No condensation） |  |  |  |  |
| IP grade | IP65 |  |  |  |  |
| ESD | Air +/-8K，Contact +/-6K |  |  |  |  |
| Surge protection | Common-Mode: 4K，Differential-Mode: 2K |  |  |  |  |

Firmware Specification

| Working mode | Access Point (Automatic WDS), Access Point (TDMA3), Site (WDS/TDMA3), Site (ARPNAT) |
| --- | --- |
| Wireless | Intelligent dynamic polling, automatic channel selection, automatic modulation mode selection, automatic transmission power control (ATPC) |
| Wireless Safety | WPA/WPA2 personal encryption, WPA/WPA2 enterprise encryption, WACL, user isolation |
| Wireless QoS | TDMA implements 4 queue priority |
| Network mode | IPv4 IPv6 |
| VLAN | Support VLAN and VLAN transparent transmission |
| WAN protocol | Static IP，DHCP client |
| Service | DHCP service, SNMP service, NTP client, route tracking, ping watchdog |
| Management | HTTP(S) GUI, SSH, SNMP read, WNMS, Telnet |
| Built Tool | Site survey, connection testing, antenna calibration |

Antenna Specification

| V Pol |  | H Pol |  |
| --- | --- | --- | --- |

| Internal antenna |  |
| --- | --- |
| Frequency range | 5180 ~5850 MHz |
| Gain | 23 dBi |
| Polarization | Dual linear |
| Cross-pol Isolation | 30 dB |
| VSWR | <1.8 |
| Azimuth beamwidth(H pol) | 6 ° |
| Azimuth beamwidth(V pol） | 6 ° |

Interface

Dimension

Packing List

| CPE | 1 |
| --- | --- |
| Dish antenna | 1 |
| PoE adapter | 1 |
| Rack kit | 1 |
| Gift Box | 1 |