11ax 3000Mbps Inwall Wireless AP

Model: GNT-AP818

Short Specification

It is a high-performance, high-speed indoor 11ax panel AP that can provide 2.4G and 5.8GHz dual band wireless services, providing a cleaner wireless environment and more stable wireless access.

Key Features:

- Simultaneously running 2.4GHz (574Mbps) and 5GHz (1201Mbps) to achieve maximum wireless throughput

- Support hardware watchdog function, automatic recovery of abnormal devices, maintenance free

- The heat sink is treated with a special surface coating, resulting in a more ideal heat dissipation effect

- Supports 48V PoE

Hardware Specification

| CPU | MT7981B+MT7976CN+RTL8211FS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Flash | Nand 128MB |  |  |  |  |
| DDR3L | 256MB |  |  |  |  |
| 2.4G Frequency | 2.4GHz ~ 2.484GHz |  |  |  |  |
| 2.4G Protocol | 802.11 b / g / n / ax |  |  |  |  |
| 5G Frequency | 5.150GHz～5.850GHz |  |  |  |  |
| 5G Protocol | 802.11 a / n / ac / ax |  |  |  |  |
| Antenna | Dual band 2*1.7dBi |  |  |  |  |
| Data Rate | 5G: 2402Mbps 2.4G:574Mbps |  |  |  |  |
| 2.4G TX Power | 802.11b | 11M | 18±2dBm | 1M | 20±2dBm |
|  | 802.11g | 54M | 16±2dBm | 6M | 18±2dBm |
|  | 802.11n HT20 | MCS7 | 15±2dBm | MCS0 | 17±2dBm |
|  | 802.11n HT40 | MCS7 | 15±2dBm | MCS0 | 17±2dBm |
|  | 802.11ax HT20 | MCS11 | 13±2dBm | MCS0 | 15±2dBm |
|  | 802.11ax HT40 | MCS11 | 13±2dBm | MCS0 | 15±2dBm |
| 5G TX Power | 802.11a | 54M | 15±2dBm | 6M | 17±2dBm |
|  | 802.11n HT20 | MCS7 | 14±2dBm | MCS0 | 16±2dBm |
|  | 802.11n HT40 | MCS7 | 14±2dBm | MCS0 | 16±2dBm |
|  | 802.11ac HT20 | MCS7 | 13±2dBm | MCS0 | 15±2dBm |
|  | 802.11ac HT40 | MCS7 | 13±2dBm | MCS0 | 15±2dBm |
|  | 802.11ac HT80 | MCS9 | 13±2dBm | MCS0 | 15±2dBm |
|  | 802.11ax HT20 | MCS11 | 12±2dBm | MCS0 | 14±2dBm |
|  | 802.11ax HT40 | MCS11 | 12±2dBm | MCS0 | 14±2dBm |
|  | 802.11ax HT80 | MCS11 | 12±2dBm | MCS0 | 14±2dBm |
|  | 802.11ax HT160 | MCS11 | 12±2dBm | MCS0 | 14±2dBm |
| 2.4G Receiving Sensitivity | 802.11b | 11M | -83dBm | 1M | -90dBm |
|  | 802.11g | 54M | -70dBm | 6M | -84dBm |
|  | 802.11n HT20 | MCS7 | -68dBm | MCS0 | -83dBm |
|  | 802.11n HT40 | MCS7 | -66dBm | MCS0 | -83dBm |
|  | 802.11ax HT20 | MCS11 | -56dBm | MCS0 | -83dBm |
|  | 802.11ax HTs40 | MCS11 | -53dBm | MCS0 | -83dBm |
| 5G Receiving Sensitivity | 802.11a | 54M | -70dBm | 6M | -83dBm |
|  | 802.11n HT20 | MCS7 | -67dBm | MCS0 | -83dBm |
|  | 802.11n HT40 | MCS7 | -64dBm | MCS0 | -83dBm |

| 5G Receiving Sensitivity | 802.11ac HT20 | MCS7 | -58dBm | MCS0 | -83dBm |
| --- | --- | --- | --- | --- | --- |
|  | 802.11ac HT40 | MCS7 | -57dBm | MCS0 | -83dBm |
|  | 802.11ac HT80 | MCS9 | -56dBm | MCS0 | -85dBm |
|  | 802.11ax HT20 | MCS11 | -57dBm | MCS0 | -83dBm |
|  | 802.11ax HT40 | MCS11 | -56dBm | MCS0 | -83dBm |
|  | 802.11ax HT80 | MCS11 | -53dBm | MCS0 | -82dBm |
|  | 802.11ax HT160 | MCS11 | -51dBm | MCS0 | -88dBm |
| 2.4G EVM | 802.11b: ≤-10 dB ；802.11g: ≤-25 dB ；802.11n: ≤-28dB ;802.11ax: ≤-35 dB |  |  |  |  |
| 5G EVM | 802.11a: ≤-25 dB ；802.11n: ≤-28 dB ；802.11ac: ≤-32 dB; 802.11ax: ≤-35 dB |  |  |  |  |
| ppm | ±20ppm |  |  |  |  |
| WAN | 1*10/100/1000M WAN, support 48V POE |  |  |  |  |
| LAN | 1*10/100/1000M LAN |  |  |  |  |
| POE | 802.3af |  |  |  |  |
| Reset | Reset to factory settings by pressing 6-10 seconds |  |  |  |  |
| Indicators | SYS |  |  |  |  |
| Max Power Consumption | <12W |  |  |  |  |
| ESD | Air:±8K, Touch:±6K |  |  |  |  |
| Electrical Surge | Common Mode: 1K, Differential Mode: 0.5K |  |  |  |  |
| Size | 86mm*86mm*38.8mm |  |  |  |  |
| Weight | 0.15kg |  |  |  |  |
| Temperature | Working: -10°C to 40°C；Storage: -40°C to 70°C；Humidity: 5％～95％(Non-Condensing) |  |  |  |  |

Firmware Specification

| Working Mode | AP(FAT/ FIT) |
| --- | --- |
| Wireless Functions | Multiple SSID functions: 2.4GHz: 4; 5.8GHz: 4. |
|  | Support SSID hidden |
|  | Support 5G Prior for a faster Ethernet. |
|  | Wireless Security: OPEN, WPA/WPA2PSK-TKIPAES, WPA3PSK-TKIPAES |
|  | Support MAC filter |
|  | Support Wi-Fi time on/off to save energy |
|  | Support client isolation to improve the wireless stability |
|  | Support RF power adjustable, adjust the RF power based on environment. |
|  | QoS: WMM |

|  | Support user quantity limited, Max 64 users to access each band. |
| --- | --- |
| Networking Function | VLAN settings |
|  | Cloud access support in FAT mode |
| Device Management | Back-up the configuration |
|  | Restore the configuration |
|  | Reset to factory default |
|  | Reboot the device: including time reboot or reboot immediately |
|  | Admin management password modify |
|  | Firmware upgrade |
|  | System log |
|  | Support firmware GUI web management, AC controller management, remote management and cloud management |

Interface

Dimension

Packing List

| AP | 1 |
| --- | --- |
| Screw | 2 |
| Gift Box | 1 |