11ax 1800Mbps Ceiling Wireless AP

Model: GNT-AP690

Short Specification

GNT-AP690 is a high-performance, high-speed indoor 11ax dual-band ceiling AP. 2.4G provides a maximum access rate of 574Mbps, and 5G provides a maximum access rate of 1201Mbps.

Key Features

- It has better transmission quality, higher transmission rate, and a maximum concurrent speed of 1775Mbps for the entire machine.

- Support hardware watchdog function, automatic recovery of abnormal devices, no maintenance required.

- Elegant appearance design, hidden port housing, and beautiful wiring.

- Both 48V PoE and DC 12V power supply modes are supported.

Hardware Specification

| Model | GNT-AP690 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CPU | MT7621AT+MT7905N+MT7975DN |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| DDR3 | 256M |  |  |  |  |
| 2.4G Band | 2.4GHz ~ 2.484GHz |  |  |  |  |
| 2.4G WIFI protocol | 802.11 b / g / n /ax |  |  |  |  |
| 5G Band | 5.150GHz～5.850GHz |  |  |  |  |
| 5G WIFI protocol | 802.11a/n/ac/ax |  |  |  |  |
| 2.4G Antenna | Built in two 2.4G antennas, each with a gain of 4.6dBi |  |  |  |  |
| 5G Antenna | Built in two 5G antennas, each with a gain of 4dBi |  |  |  |  |
| Maximum rate | 2.4G up to 574Mbps, 5G up to 1201Mbps |  |  |  |  |
| 2.4G Tx power | 802.11b | 11M | 22±2dBm | 1M | 22±2dBm |
|  | 802.11g | 54M | 19±2dBm | 6M | 20±2dBm |
|  | 802.11n HT20 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11n HT40 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT20 | MCS11 | 15±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT40 | MCS11 | 15±2dBm | MCS0 | 20±2dBm |
| 5G Tx power | 802.11a | 54M | 18±2dBm | 6M | 20±2dBm |
|  | 802.11n HT20 | MCS7 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11n HT40 | MCS7 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11ac HT20 | MCS7 | 16±2dBm | MCS0 | 19±2dBm |
|  | 802.11ac HT40 | MCS7 | 16±2dBm | MCS0 | 19±2dBm |
|  | 802.11ac HT80 | MCS9 | 16±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT20 | MCS11 | 15±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT40 | MCS11 | 15±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT80 | MCS11 | 15±2dBm | MCS0 | 19±2dBm |
| 2.4G Receiving sensitivity | 802.11b | 11M | -82dBm | 1M | -88dBm |
|  | 802.11g | 54M | -72dBm | 6M | -88dBm |
|  | 802.11n HT20 | MCS7 | -70dBm | MCS0 | -88dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -85dBm |

|  | 802.11ax HT20 | MCS11 | -60dBm | MCS0 | -88dBm |
| --- | --- | --- | --- | --- | --- |
|  | 802.11ax HT40 | MCS11 | -56dBm | MCS0 | -85dBm |
| 5G Receiving sensitivity | 802.11a | 54M | -72dBm | 6M | -89dBm |
|  | 802.11n HT20 | MCS7 | -71dBm | MCS0 | -89dBm |
|  | 802.11n HT40 | MCS7 | -68dBm | MCS0 | -86dBm |
|  | 802.11ac HT20 | MCS7 | -66dBm | MCS0 | -89dBm |
|  | 802.11ac HT40 | MCS7 | -60dBm | MCS0 | -86dBm |
|  | 802.11ac HT80 | MCS9 | -57dBm | MCS0 | -83dBm |
|  | 802.11ax HT20 | MCS11 | -58dBm | MCS0 | -88dBm |
|  | 802.11ax HT40 | MCS11 | -55dBm | MCS0 | -85dBm |
|  | 802.11ax HT80 | MCS11 | -52dBm | MCS0 | -82dBm |
| 2.4G EVM | 802.11b:≤-10 dB；802.11g:≤-25 dB；802.11n:≤-28dB ; 802.11ax: ≤-35 dB |  |  |  |  |
| 5G EVM | 802.11a:≤-25 dB； 802.11n:≤-28 dB；802.11ac:≤-32 dB ; 802.11ax:≤-35 dB |  |  |  |  |
| frequency deviation（ppm） | ±20ppm |  |  |  |  |
| Port (WAN) | 1*10/100/1000M，support 48V POE |  |  |  |  |
| Port（LAN） | 1*10/100/1000M |  |  |  |  |
| Reset Button | reset（Long press for 6-10 seconds to restore factory default configuration） |  |  |  |  |
| LED | Tricolor lamp（Three different colors represent：sys-Red，2.4G-Green，5.8G-Blue） |  |  |  |  |
| Supply | DC2.0 12V 1.5A，PoE 802.3af |  |  |  |  |
| Consumption | < 15W |  |  |  |  |
| Size | 186mm × 186mm ×36mm |  |  |  |  |
| Height | 0.4kg |  |  |  |  |
| Environment | Operating temperature-10°C to 40°C；Storage temperature: -40°C to 70°C；humidity：5％～95％（No condensation） |  |  |  |  |
| ESD | Air +/-8K， Contact +/-4K |  |  |  |  |

Firmware Specification

| Working Mode | Gateway, AP |
| --- | --- |
| Wireless Functions | Multiple SSID functions: 2.4GHz: 4; 5.8GHz: 4. |
|  | Support SSID hidden |
|  | Support seamless roaming, 802.11kvr standard. |
|  | Support 5G Prior for a faster Ethernet. |
|  | Wireless Security: Open, WPA, WPA2PSK_TKIPAES, WAP2_EAP, 802.1x |
|  | Support MAC filter |
|  | Support Wi-Fi time on/off to save energy |
|  | Support client isolation to improve the wireless stability |
|  | Support RF power adjustable, adjust the RF power based on environment. |
|  | Support user quantity limited, Max 64 users to access each band. |
| Networking Function | VLAN settings |
|  | Cloud access support in gateway mode |
| Device Management | Back-up the configuration |
|  | Restore the configuration |
|  | Reset to factory default |
|  | Reboot the device: including time reboot or reboot immediately |
|  | Admin management password modify |
|  | Firmware upgrade |
|  | System log |
|  | Support firmware GUI web management, AC controller management, remote management and cloud management |
| Protocols | IPv4 |

Interface

Dimension:

Packing List

| AP | 1 |
| --- | --- |
| Lan cable | 1 |
| Mounting Accessories | 1 |
| Gift Box | 1 |