11ax 3000Mbps Ceiling Wireless AP

Model: GNT-AP835F

Short Specification

GNT-AP835F is a high-performance dual-band 802.11AX product that provides 2.4G and 5.8GHz dual-band wireless services. 2.4G provides a maximum access rate of 574Mbps, and 5G provides a maximum access rate of 2402Mbps.

Main Features:

- It has better transmission quality and higher transmission rate. The concurrent rate of the whole machine can reach up to 3000Mbps.

- Support hardware watchdog function, automatic recovery of abnormal equipment, maintenance-free.

- Elegant appearance design, hidden port housing, beautiful wiring.

- The addition of lightning protection board components greatly enhances the protection capability of the equipment.

Hardware Specification

| Model | GNT-AP835F |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CPU | MT7981B+MT7976DA+GPY211 |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| DDR3 | 256M |  |  |  |  |
| 2.4G Frequency | 2.4GHz – 2.484GHz |  |  |  |  |
| 2.4G Protocol | 802.11 b / g / n/ac /ax |  |  |  |  |
| 5G Frequency | 5.150GHz～5.825GHz |  |  |  |  |
| 5G Protocol | 802.11 a / n / ac /ax |  |  |  |  |
| Dual Band Antenna | 4*4dBi |  |  |  |  |
| Data Rate | 2.4G: 574Mbps; 5G: 2402Mbps |  |  |  |  |
| 2.4G TX Power | 802.11b | 11M | 22±2dBm | 1M | 22±2dBm |
|  | 802.11g | 54M | 19±2dBm | 6M | 21±2dBm |
|  | 802.11n HT20 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11n HT40 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT20 | MCS11 | 16±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT40 | MCS11 | 16±2dBm | MCS0 | 20±2dBm |
| 5G TX Power | 802.11a | 54M | 19±2dBm | 6M | 21±2dBm |
|  | 802.11n HT20 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11n HT40 | MCS7 | 18±2dBm | MCS0 | 20±2dBm |
|  | 802.11ac HT20 | MCS7 | 17±2dBm | MCS0 | 20±2dBm |
|  | 802.11ac HT40 | MCS7 | 17±2dBm | MCS0 | 20±2dBm |
|  | 802.11ac HT80 | MCS9 | 17±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT20 | MCS11 | 16±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT40 | MCS11 | 16±2dBm | MCS0 | 20±2dBm |
|  | 802.11ax HT80 | MCS11 | 16±2dBm | MCS0 | 19±2dBm |
|  | 802.11ax HT160 | MCS11 | 14±2dBm | MCS0 | 18±2dBm |
| 2.4G Receiving Sensitivity | 802.11b | 11M | -87dBm | 1M | -96dBm |
|  | 802.11g | 54M | -75dBm | 6M | -93dBm |
|  | 802.11n HT20 | MCS7 | -73dBm | MCS0 | -92dBm |

|  | 802.11n HT40 | MCS7 | -70dBm | MCS0 | -89dBm |
| --- | --- | --- | --- | --- | --- |
|  | 802.11ax HT20 | MCS11 | -64dBm | MCS0 | -92dBm |
|  | 802.11ax HT40 | MCS11 | -61dBm | MCS0 | -89dBm |
| 5G Receiving Sensitivity | 802.11a | 54M | -74dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -73dBm | MCS0 | -90dBm |
|  | 802.11n HT40 | MCS7 | -70dBm | MCS0 | -87dBm |
|  | 802.11ac HT20 | MCS7 | -67dBm | MCS0 | -91dBm |
|  | 802.11ac HT40 | MCS7 | -63dBm | MCS0 | -88dBm |
|  | 802.11ac HT80 | MCS9 | -60dBm | MCS0 | -84dBm |
|  | 802.11ax HT20 | MCS11 | -62dBm | MCS0 | -91dBm |
|  | 802.11ax HT40 | MCS11 | -59dBm | MCS0 | -89dBm |
|  | 802.11ax HT80 | MCS11 | -56dBm | MCS0 | -86dBm |
|  | 802.11ax HT160 | MCS11 | -53dBm | MCS0 | -83dBm |
| 2.4G EVM | 802.11b:≤-10 dB；802.11g:≤-25 dB；802.11n:≤-28dB ; 802.11ax: ≤-35 dB |  |  |  |  |
| 5G EVM | 802.11a:≤-25 dB； 802.11n:≤-28 dB；802.11ac:≤-32 dB ; 802.11ax:≤-35 dB |  |  |  |  |
| ppm | ±20ppm |  |  |  |  |
| WAN | 1*10/100/1000/2500Mbps WAN, support 48V PoE |  |  |  |  |
| LAN | 1*10/100/1000Mbps LAN |  |  |  |  |
| Reset | Reset to factory settings by pressing 6-10 seconds |  |  |  |  |
| Indicators | PWR, SYS, WAN, 2.4G, 5.8G |  |  |  |  |
| Power | PoE 802.3at |  |  |  |  |
| Max Power Consumption | <18W |  |  |  |  |
| Dimension | 304mm×181mm×88mm |  |  |  |  |
| Weight | 1.6kg |  |  |  |  |
| Temperature | Working: -20°C to 55°C；Storage: -40°C to 70°C；Humidity: 5％～95％(Non-Condensing) |  |  |  |  |
| ESD | Air:±8K, Touch:±6K |  |  |  |  |
| Electrical Surge | Common Mode: 4K, Differential Mode: 2K |  |  |  |  |

Firmware Specification

| Working Mode | Gateway, AP |
| --- | --- |
| Wireless Functions | Multiple SSID functions: 2.4GHz: 4; 5.8GHz: 4. |
|  | Support SSID hidden |
|  | Support seamless roaming |
|  | Support 5G Prior for a faster Ethernet. |
|  | Wireless Security: Open, WPA, WPA2PSK_TKIPAES, WAP2_EAP, WPA3 |
|  | Support MAC filter |
|  | Support Wi-Fi time on/off to save energy |
|  | Support client isolation to improve the wireless stability |
|  | Support RF power adjustable, adjust the RF power based on environment. |
| Networking Function | VLAN settings |
|  | Cloud access support in gateway mode |
| Device Management | Back-up the configuration |
|  | Restore the configuration |
|  | Reset to factory default |
|  | Reboot the device: including time reboot or reboot immediately |
|  | Admin management password modify |
|  | Firmware upgrade |
|  | System log |
|  | Support firmware GUI web management, AC controller management, remote management and cloud management |
| Protocols | IPv4 |

Interface

Dimension

Packing List

| AP | 1 |
| --- | --- |
| Lan cable | 1 |
| Mounting Accessories | 1 |
| Gift Box | 1 |